CSE 396 – Computer Engineering Project

ASSIGNMENT 2

Module Interface & Header Design

LIDAR-Guided Hospital Navigation Robot

Module 4: SLAM – Mapping & Localization

Group 10

Submission Date: 2025-03-24

| Name | Student ID | Role |
| --- | --- | --- |
| Fatih Emre OĞAN | — | Primary Lead |
| Mervan Deniz YILDIRIM | — | Primary Lead |
| Ertuğrul ERGÜL | — | Secondary Support |
| Hicran KOÇ | — | Secondary Support |

# 1. Module – Student Matching Table

| Module ID | Module Name | Responsible Student(s) | Role | Brief Role |
| --- | --- | --- | --- | --- |
| MOD-01 | Voice Communication | Gökhan (P), Fatih (S), Hicran (S) | Primary/Sec. | Speech-to-text, TTS output |
| MOD-02 | Digital Twin & Visualization | Samet (P), Gökhan (S), Mervan (S) | Primary/Sec. | Gazebo/Unity 3D visualization |
| MOD-03 | Hardware & Motor Control | Hicran (P), Mervan (S), Sayed (S) | Primary/Sec. | PWM, encoders, IMU, UART |
| MOD-04 | SLAM – Mapping & Localization | Fatih (P), Mervan (P), Ertuğrul (S), Hicran (S) | Primary | ICP-based pose estimation, 2D occupancy map |
| MOD-05 | NAV – Path Planning & Control | Ertuğrul (P), Ömer (P), Fatih (S) | Primary/Sec. | A*, waypoint following, FSM |
| MOD-06 | Sensor Integration & Control | Sayed (P), Gökhan (S), Ömer (S) | Primary/Sec. | Ultrasonic, IR, LiDAR raw data bus |

# 2. System / Module Overview

## 2.1 Overall Architecture

The system is a differential-drive autonomous indoor robot running entirely on a Raspberry Pi 4. Six software modules handle voice I/O, sensor fusion, localization, path planning, motor control, and 3-D visualization. All inter-module communication occurs over in-process shared memory (Python objects / C structs) on the RPi; physical hardware I/O is managed by Module 3.

## 2.2 Module 4 – SLAM: Purpose & Responsibilities

Module 4 implements the Simultaneous Localization and Mapping (SLAM) pipeline. It is the authoritative source of the robot's estimated pose (x, y, θ) and the 2-D occupancy-grid map of the hospital floor.

- Receives raw LiDAR scan frames (slam_scan_t) from Module 6 at 10 Hz.

- Receives odometry deltas (dx, dy, dtheta) from Module 3 every ~50 ms.

- Runs KISS-ICP scan-to-map matching to correct accumulated drift.

- Maintains a 400×400 cell occupancy grid at 5 cm resolution covering ~400 m².

- Publishes the corrected pose to Module 5 (Navigation) via callback and slam_get_pose().

- Publishes the updated map to Module 5 and Module 2 (Digital Twin).

## 2.3 Block Diagram (Text Representation)

M6 Sensor ──── slam_scan_t (10 Hz) ────► ┌──────────────────────┐ │ M4 SLAM MODULE │ M3 Hardware ─── dx,dy,dθ (50 ms) ────────►│ ICP + Occupancy Grid│ └──────────┬───────────┘ slam_pose_t ┴ slam_map_t* ┌────────────┴─────────────┐ ▼ ▼ M5 NAV Module M2 Digital Twin

# 3. Hardware & Platform Summary

| Component | Model / Detail | Relevance to M4 |
| --- | --- | --- |
| Main Computer | Raspberry Pi 4 (2 GB+), Raspberry Pi OS | Runs all SLAM computations, Python 3 + KISS-ICP |
| LiDAR Sensor | Slamtec RPLidar A2 (360°, 8 m range, 8 kHz) | Primary input for scan matching and map building |
| IMU | InvenSense MPU-6050 (I²C) | Heading correction via complementary filter (α=0.98) |
| Wheel Encoders | Rotary encoders on DC gear motors (GPIO) | Odometry delta source for ICP prior seeding |
| Motor Driver | L298N dual H-bridge | Indirect: M4 output drives M5 which drives M3 |

## 3.1 Communication Buses Used by M4

- I²C: MPU-6050 IMU @ 400 kHz (via M3 Hardware module)

- USB Serial: RPLidar A2 UART @ 115200 baud (via M6 Sensor module)

- GPIO: Encoder pulse reading (via M3 Hardware module)

## 3.2 External Libraries

- kiss-icp (v0.3+) — KISS-ICP scan matching, Python/C++: kiss_icp_ros2

- numpy — array operations for occupancy grid management

- RPi.GPIO — low-level GPIO access (used by M3, data passed to M4)

# 4. Inter-Module Communication

The table below describes every data exchange that Module 4 (SLAM) participates in as sender or receiver.

| From | To | Data / Signal | Type | Direction | Notes / Frequency |
| --- | --- | --- | --- | --- | --- |
| M6 | M4 | slam_scan_t | struct | M6 → M4 | 10 Hz; full 360° LiDAR scan frame; shared memory |
| M3 | M4 | float dx, dy, dtheta | float×3 | M3 → M4 | ~50 ms; odometry deltas per control cycle; UART / shared mem |
| M4 | M5 | slam_pose_t | struct | M4 → M5 | 10 Hz via slam_pose_cb_t callback or slam_get_pose() poll |
| M4 | M5 | const slam_map_t * | ptr to struct | M4 → M5 | On map update via slam_map_cb_t; M5 must copy if persistent |
| M4 | M2 | slam_pose_t, slam_map_t* | struct / ptr | M4 → M2 | 10 Hz; for live 3-D digital twin rendering |

# 5. Known Risks & Open Questions

| Risk / Open Question | Likelihood | Impact | Mitigation / Status |
| --- | --- | --- | --- |
| ICP localization drift during long runs (>20 m) | Medium | High | Tune ICP threshold; add periodic pose-snap at corridor junctions |
| KISS-ICP latency may exceed 100 ms on RPi 4 | Medium | High | Benchmark early; fall back to pure odometry if needed; consider C++ binding |
| slam_get_map() pointer invalidated by concurrent slam_update() | High | Medium | Define copy contract with M5 and M2; add mutex or double-buffer |
| Occupancy map resolution (5 cm) may be too coarse for narrow corridors | Low | Medium | Evaluate during site survey; SLAM_MAP_RESOLUTION_M is a macro and can be reduced |
| RPLidar A2 USB instability on RPi causing scan gaps | Low | High | Test early; use powered USB hub; add retry logic in M6 |
| Interface between M4 and M5: is pose callback sufficient or does M5 need a pull API? | — | — | Open question — M4 and M5 leads to agree by W7 |

# 6. Header File — slam.h (Excerpt)

The full header file is submitted as slam.h alongside this report. The excerpt below highlights the key data types and function signatures.

/* slam.h – Module 4: SLAM (v0.1, 2025-03-24) */ /* Authors: Fatih Emre OGAN ¦ Mervan Deniz YILDIRIM */ typedef enum { SLAM_OK=0, SLAM_ERR_NOT_INIT=-1, ... } slam_status_t; typedef struct { float x, y, theta; uint32_t timestamp; } slam_pose_t; typedef struct { uint8_t cells[400][400]; float origin_x, origin_y, resolution; uint32_t timestamp; } slam_map_t; slam_status_t slam_init(const slam_pose_t *initial_pose); slam_status_t slam_update(const slam_scan_t *scan, float dx, float dy, float dtheta); slam_status_t slam_get_pose(slam_pose_t *out); slam_status_t slam_get_map(const slam_map_t **out); slam_status_t slam_register_pose_callback(slam_pose_cb_t cb); slam_status_t slam_register_map_callback(slam_map_cb_t cb); slam_status_t slam_get_diagnostics(slam_diagnostics_t *out); slam_status_t slam_reset(void);