Computer Engineering Project Course ASSIGNMENT 2 Module Interface & Header Design

1. Overview & Motivation

Your project is divided into independently developed software/hardware modules, each owned by a different student or sub-team. Modules must cooperate by exchanging data, commands, and status information through well-defined interfaces. Before any coding begins, every team must agree on what those interfaces look like — and document that agreement formally.

This assignment is your team's first engineering contract. A clear, thorough interface specification prevents integration surprises later in the semester and forces you to think carefully about system architecture early on.

|  | Note: Interfaces will evolve as the project progresses — that is expected. What matters is that changes are communicated to all affected modules and the header files are versioned accordingly. |
| --- | --- |

2. Deliverables

You must submit three types of deliverables:

| # | Deliverable | Format | Notes |
| --- | --- | --- | --- |
| 1 | C/C++ Header Files — one per module interface | .h / .hpp | See Section 4 for required contents |
| 2 | PDF Report — project interface documentation | .pdf | See Section 3 for required contents |
| 3 | README / Markdown — module quick-reference | .md | One file per module; see Section 5 |

3. PDF Report Contents

3.1  Cover Page

- Course name, assignment title, submission date

- Project name / title

- Team member names, student IDs, and assigned module(s)

3.2  Module–Student Matching Table

Provide a clear table showing which student(s) are responsible for each module.

| Module ID | Module Name | Responsible Student(s) | Brief Role |
| --- | --- | --- | --- |
| MOD-01 | e.g. Sensor Driver | Student A | Reads ADC, exposes get_reading() |
| MOD-02 | e.g. Control Logic | Student B | PID loop, consumes sensor data |
| MOD-03 | e.g. Communication | Student C | UART/SPI bridge to host PC |
| ... | ... | ... | ... |

3.3  System / Module Overview

- A block diagram showing all modules and the data flows between them

- A short paragraph (3–5 sentences) describing each module's purpose

- Overall system architecture: microcontroller(s), FPGA, host PC, embedded OS, etc.

3.4  Hardware & Platform Summary

- Target hardware (MCU model, dev board, sensor models, actuator models)

- Communication buses used (I2C, SPI, UART, CAN, USB, Ethernet, ...)

- Clock frequencies, voltage levels, timing constraints (if known)

- External libraries or SDKs required (e.g. STM32 HAL, FreeRTOS, Arduino core)

3.5  Inter-Module Communication Description

For every pair of modules that exchange data, describe the communication in detail. Be as specific as possible — this is the main engineering content of the report.

| From | To | Data / Signal | Type | Direction | Notes / Frequency |
| --- | --- | --- | --- | --- | --- |
| MOD-01 | MOD-02 | sensor_data_t | struct | 01 -> 02 | 100 Hz, shared memory queue |
| MOD-02 | MOD-03 | cmd_packet_t | struct | 02 -> 03 | On change, UART DMA |
| MOD-03 | MOD-02 | ack_t | enum | 03 -> 02 | ACK/NACK after each command |

3.6  Known Risks & Open Questions

- List any interface decisions that are still uncertain

- Identify hardware items not yet procured or confirmed

- Note any timing or resource constraints that might require renegotiation

4. C/C++ Header File Requirements

Each module must provide at least one .h (or .hpp) header file that fully describes its public interface. Consumers of the module should be able to understand and use the interface by reading the header alone — without reading the implementation.

4.1  Mandatory Sections in Every Header

| Element | Requirement | Example |
| --- | --- | --- |
| Include guard | Prevent multiple inclusion | #ifndef MODULE_SENSOR_H / #define MODULE_SENSOR_H |
| File header comment | Module name, author, date, version, brief description | /** @file sensor.h @brief ADC driver @version 0.1 */ |
| Includes / dependencies | All required headers; system vs project includes | #include <stdint.h> / #include "project_types.h" |
| Constants & macros | All #defines or const values the consumer needs | #define SENSOR_MAX_SAMPLES 64 |
| Data types & structs | typedef / struct / enum for all exchanged data types | typedef struct { uint16_t raw; float voltage; } sensor_data_t; |
| Public function declarations | Every function other modules will call; Doxygen comment on each | int sensor_init(void); with @brief @return |
| Callback / ISR signatures (if applicable) | Any callback typedefs or ISR prototypes the consumer must register | typedef void (*sensor_cb_t)(sensor_data_t *); |

4.2  Version & Change Log

Include a version string and a brief change-log comment block at the top of each header. When you update an interface, bump the version number and add an entry.

/* Changelog:

* v0.1 (2025-03-01) - Initial draft, sensor_init / sensor_read

* v0.2 (2025-03-14) - Added sensor_cb_t callback type

*/

4.3  Naming Conventions

- modulename_FunctionName()Use a consistent module prefix for all symbols:

- Types end in _t  — e.g. sensor_data_t, motor_cmd_t

- SENSOR_ERR_TIMEOUTEnum members are ALL_CAPS with module prefix:

- SENSOR_MAX_SAMPLESConstants / macros are ALL_CAPS:

5. README / Markdown File Requirements

Each module must include a README.md file alongside its header file(s). This file serves as a quick-reference guide for anyone integrating with the module.

5.1  Mandatory Sections in Every README

- Module name and one-sentence purpose

- Author name and student ID

- Dependencies — which other modules or external libraries must be present

- Quick-start integration example — minimal code snippet showing how to call the module

- API summary — a table listing every public function, its parameters, and return value

- Known limitations and TODOs

- Version history (mirrors the header changelog)

5.2  Format Rules

- Use standard Markdown (GitHub-flavoured): # headings, ** bold **, backtick code blocks

- The first line must be: # MODULE_NAME — Module Display Name

- Code snippets must use fenced code blocks with the language tag: ```c or ```cpp

6. Example Header Skeleton

The skeleton below shows the expected structure. Your header should follow this layout with content appropriate to your module.

#ifndef MODULE_SENSOR_H

#define MODULE_SENSOR_H

/**

* @file    sensor.h

* @brief   ADC Sensor Driver - public interface

* @author  [Your Name]  [Student ID]

* @date    [YYYY-MM-DD]

* @version 0.1

*

* Changelog:

*   v0.1 - Initial draft

*/

#include <stdint.h>

#include "project_types.h"   /* shared project-wide types */

/* -- Constants -------------------------------------------- */

#define SENSOR_MAX_SAMPLES   64

#define SENSOR_TIMEOUT_MS   100

/* -- Data Types -------------------------------------------- */

typedef enum {

SENSOR_OK          =  0,

SENSOR_ERR_INIT    = -1,

SENSOR_ERR_TIMEOUT = -2

} sensor_status_t;

typedef struct {

uint16_t raw;        /**< raw 12-bit ADC value  */

float    voltage;    /**< converted voltage (V) */

uint32_t timestamp;  /**< ms since boot         */

} sensor_data_t;

/* -- Callback Type ---------------------------------------- */

typedef void (*sensor_cb_t)(const sensor_data_t *data);

/* -- Public Functions ------------------------------------- */

/** @brief  Initialise sensor hardware.

*  @return SENSOR_OK on success, error code otherwise */

sensor_status_t sensor_init(void);

/** @brief  Start continuous sampling.

*  @param  hz  Sampling rate (1-1000)

*  @param  cb  Callback on each sample (may be NULL)

*  @return SENSOR_OK on success */

sensor_status_t sensor_start(uint16_t hz, sensor_cb_t cb);

/** @brief  Read the latest sample synchronously.

*  @param  out  Pointer to caller-owned sensor_data_t

*  @return SENSOR_OK or SENSOR_ERR_TIMEOUT */

sensor_status_t sensor_read(sensor_data_t *out);

/** @brief  Stop sampling and power down the peripheral. */

void sensor_stop(void);

#endif /* MODULE_SENSOR_H */

7. Submission Checklist

|  | Before submitting, verify every item below is completed. Incomplete submissions will be returned without grading. |
| --- | --- |

- PDF report includes a cover page with all team members and module assignments

- Module–student matching table is complete and unambiguous

- Block diagram shows all modules with data-flow arrows labelled

- Hardware and platform section names the exact MCU / board / sensor models

- Inter-module communication table covers every module pair that exchanges data

- One header file (.h / .hpp) provided per module interface

- Every header has an include guard

- Every header has a file-level comment (module name, author, date, version)

- Every public function has a Doxygen comment with @brief, @param, @return

- All data types shared between modules are declared in a header (no magic numbers)

- Naming conventions are consistent throughout (prefix, _t suffix, ALL_CAPS macros)

- Version string and changelog comment block present in each header

- README.md present for every module with all required sections

- README API summary table matches the header declarations exactly

- Risks and open questions section has at least two entries

- All files are submitted in a single compressed archive named: [TeamName]_Assignment1.zip

Questions? Post to the course forum or bring them to office hours. Interface design is an iterative process — your first draft does not need to be perfect, but it does need to be thoughtful.