1

Application

Operating System

Digital Design Circuit Design

Software

Compiler Instruction Set Instr. Set Processor

I/O system Architecture

Datapath & Control

Hardware

Layout

► The repertoire of instructions of a computer

► Different computers have different instruction sets

◦ But with many aspects in common

► Early computers had very simple instruction sets

◦ Simplified implementation

► Many modern computers also have simple instruction sets

► Used as the example throughout the book

► Stanford MIPS commercialized by MIPS Technologies (www.mips.com)

► Large share of embedded core market

◦ Applications in consumer electronics, network/storage equipment, cameras, printers, ...

► Typical of many modern ISAs

◦ See MIPS Reference Data tear-out card, and Appendixes B and E

... ... Variable:

Fixed:

Hybrid:

► Memory to Memory Machines

◦ We need storage for temporaries

◦ Memory is slow

◦ Memory is big (lots of address bits)

► Architectural Registers

◦ Registers can hold temporary variables

◦ Registers are faster than memory

◦ Memory traffic is reduced, so program is speed up (since registers are faster than memory)

◦ Code density improves (since register named with fewer bits than memory location)

► Stack (not a register file but an operand stack)

◦ 0 address add tos = tos + next

► Accumulator (1 register):

◦ 1 address add A acc = acc + mem[A]

► General Purpose Register File (Register-Memory):

◦ 2 address add A B EA(A) = EA(A) + EA(B)

◦ 3 address add A B C EA(A) = EA(B) + EA(C)

► General Purpose Register File (Load/Store):

◦ 3 address add Ra Rb Rc Ra = Rb + Rc

◦ load Ra Rb Ra = mem[Rb]

◦ store Ra Rb mem[Rb] = Ra

► Comparison:

◦ Bytes per instruction? Number of Instructions? Cycles per instruction?

► Code sequence for C = A + B for four classes of instruction sets:

Stack Accumulator Register Register

(register-memory) (load-store) Push A Load A Load R1, A Load R1, A Push B Add B Add R1, B Load R2 ,B Add Store C Store C, R1 Add R3, R1, R2 Pop C Store C, R3

MIPS is one of these: this is what we’ll be learning

► The MIPS architecture is considered to be a typical RISC architecture.

◦ Simplified instruction set => easier to study

◦ Most new machines are RISC

► Programmable storage

◦ 31 x 32-bit GPRs (r0 = 0)

◦ special purpose - HI, LO, PC

◦ 32 x 32-bit FP regs

◦ 2^32 x bytes of memory

r0 r1 ° ° ° r31 PC lo hi

f0 f1 ° ° ° f31

► With MIPS, there is a convention for mapping register names into general purpose register numbers.

Name Register Address

Usage Preserved

on call? $zero 0 the constant value 0 n.a. $v0-$v1 2-3 values for result and expression evaluation no $a0-$a3 4-7 arguments no $t0-$t7 8-15 temporaries no $s0-$s7 16-23 saved yes $t8-$t9 24-25 more temporaries no $gp 28 global pointer yes $sp 29 stack pointer yes $fp 30 frame pointer yes $ra 31 return address yes

► Arithmetic instructions have 3 operands

◦ Two sources and one destination

add a, b, c # a gets b + c

► Operand order is fixed (destination first)

Example:

C code: A = B + C

MIPS code: add $s0, $s1, $s2

(associated with variables by compiler)

► Design Principle: Simplicity favours regularity

◦ Regularity makes implementation simpler

◦ Simplicity enables higher performance at lower cost

► Of course this complicates some things...

C code: A = B + C + D; E = F - A;

MIPS code: add $t0, $s1, $s2 add $s0, $t0, $s3 sub $s4, $s5, $s0

► Operands must be registers, only 32 registers provided

► Design Principle: Smaller is faster.

a = b + c – d;

Compiler

add R1, R2, R3 sub R5, R1, R4

Assembler

0100111001001010 1000001010001011

Register file Program counter

Instruction register

ALU

Control logic Memory address register

Memory data register

Executable (binary)

instr $rd, $rs, $rt

op rs rt rd shamt funct

6 bits 6 bits 5 bits 5 bits 5 bits 5 bits

► Instruction fields

◦ op: operation code (opcode)

◦ rs: first source register number

◦ rt: second source register number

◦ rd: destination register number

◦ shamt: shift amount (00000 for now)

◦ funct: function code (extends opcode)

op rs rt rd shamt funct

6 bits 6 bits 5 bits 5 bits 5 bits 5 bits

add $t0, $s1, $s2

special $s1 $s2 $t0 0 add

0 17 18 8 0 32

000000 10001 10010 01000 00000 100000

0000 0010 0011 0010 0100 0000 0010 00002 = 0232 402016

► Base 16

◦ Compact representation of bit strings

◦ 4 bits per hex digit

0 0000 4 0100 8 1000 c 1100 1 0001 5 0101 9 1001 d 1101 2 0010 6 0110 a 1010 e 1110 3 0011 7 0111 b 1011 f 1111

∎ Example: eca8 6420

∎ 1110 1100 1010 1000 0110 0100 0010 0000

► Registers are faster to access than memory

► Arithmetic instructions operands must be registers,

— only 32 registers provided

► Operating on memory data requires loads and stores

◦ More instructions to be executed

► Compiler associates variables with registers

► What about programs with lots of variables ??

Control

Input

Memory

Datapath

Output

Processor I/O

► Viewed as a large, single-dimension array, with an address.

► A memory address is an index into the array

► "Byte addressing" means that the index points to a byte of memory.

0 1 2 3 4 5 6 ...

8 bits of data 8 bits of data

8 bits of data

8 bits of data

8 bits of data

8 bits of data

8 bits of data

► Bytes are nice, but most data items use larger "words“

► For MIPS, a word is 32 bits or 4 bytes.

0 32 bits of data 4 32 bits of data 8 32 bits of data 12

32 bits of data

► 232 bytes with byte addresses from 0 to 232-1

► 230 words with byte addresses 0, 4, 8, ... 232 - 4

0 1 2 3

► Words are aligned

Aligned i.e., what are the least 2 significant bits of a word address?

Not aligned

Alignment: require that objects fall on address that is multiple of their size.

► Big Endian: address of most significant byte = word address (xx00 = Big End of word)

◦ IBM 360/370, Motorola 68k, MIPS, Sparc, HP PA

► Little Endian: address of least significant byte = word address (xx00 = Little End of word)

◦ Intel 80x86, DEC Vax, DEC Alpha

little endian byte 0 3 2 1 0 msb lsb

0 1 2 3 big endian byte 0

► C code: g = h + A[8];

◦ g in $s1, h in $s2, base address of A in $s3

► Compiled MIPS code:

◦ Index 8 requires offset of 32?  4 bytes per word

lw $t0, 32($s3) # $t0 <= Mem[$s3 + 32] add $s1, $s2, $t0

offset base register

► Arrays are often stored in memory - why?

► Replace the C code for A[11] = A[10] + b by equivalent MIPS instructions.

► Assume b is in register $s5, the starting address for array A is in $s6, using and 32-bit integer data. Instruction Comment lw $t3, 40($s6) $t3 = A[10] add $t4, $t3, $s5 $t4 = A[10] + b sw $t4, 44($s6) A[11] = $t4

► Why are array indices multiplied by 4?

16 6 bits 5 bits 5 bits bits I-type op rs rt immed ► Instruction fields

◦ op: operation code (opcode)

◦ rs: first source register number

◦ rt: second source register number

◦ immed: immediate value

lw $t0, 32($s3) # $t0 <= Mem[$s3 + 32]

35 19 8 32

► MIPS — loading words but addressing bytes

— arithmetic on registers only

Instruction Meaning

add $s1, $s2, $s3 $s1 = $s2 + $s3 sub $s1, $s2, $s3 $s1 = $s2 – $s3 lw $s1, 100($s2) $s1 = mem[$s2+100] sw $s1, 100($s2) mem[$s2+100] = $s1

► Small constants are used quite frequently (50% of operands) e.g., A = A + 5; B = B + 1; C = C - 18;

► MIPS Instructions:

addi $29, $29, 4 slti $8, $18, 10 andi $29, $29, 6 ori $29, $29, 4

► How do we make this work?

16 bits 5 bits 5 bits 6 bits

immed rt rs op I-type

► Constant data specified in an instruction addi $s1, $s3, 4 ($s1 <= $s3 + 4)

8 19 17 4

► No subtract immediate instruction

◦ Just use a negative constant addi $s2, $s1, -1

► Design Principle 3: Make the common case fast

◦ Small constants are common

◦ Immediate operand avoids a load instruction

Instruction Example Meaning Comments add add $1,$2,$3 $1 = $2 + $3 3 operands; exception possible subtract sub $1,$2,$3 $1 = $2 – $3 3 operands; exception possible add immediate addi $1,$2,100 $1 = $2 + 100 + constant; exception possible add unsigned addu $1,$2,$3 $1 = $2 + $3 3 operands; no exceptions subtract unsigned subu $1,$2,$3 $1 = $2 – $3 3 operands; no exceptions add imm. unsign. addiu $1,$2,100 $1 = $2 + 100 + constant; no exceptions multiply mult $2,$3 Hi, Lo = $2 x $3 64-bit signed product multiply unsigned multu $2,$3 Hi, Lo = $2 x $3 64-bit unsigned product divide div $2,$3 Lo = $2 ÷ $3, Lo = quotient, Hi = remainder

Hi = $2 mod $3 divide unsigned divu $2,$3 Lo = $2 ÷ $3, Unsigned quotient &

Hi = $2 mod $3 remainder Move from Hi mfhi $1 $1 = Hi Used to get copy of Hi Move from Lo mflo $1 $1 = Lo Used to get copy of Lo

Note: Move from Hi and Move from Lo are really data transfers

► Why not blt, bge, etc?

► Hardware for <, ≥, ... slower than =, ≠

◦ Combining with branch involves more work per instruction, requiring a slower clock

◦ All instructions penalized!

► beq and bne are the common case

► This is a good design compromise

► Perform multiply, divide

◦ mult rs, rt

◦ multu rs, rt

◦ div rs, rt

◦ divu rs, rt

► Move result from multiply, divide

◦ mfhi rd

◦ mflo rd

Registers

Mult/Div Unit

HI LO

► Instructions for bitwise manipulation

Operation C MIPS Shift left << sll Shift right >> srl Bitwise AND & and, andi Bitwise OR | or, ori

∎ Useful for extracting and inserting

groups of bits in a word

op rs rt rd shamt funct

6 bits 6 bits 5 bits 5 bits 5 bits 5 bits

► shamt: how many positions to shift

► Shift left logical

◦ Shift left and fill with 0 bits

◦ sll by i bits multiplies by 2i

◦ Ex: sll $t0, $t0, 5

► Shift right logical

◦ Shift right and fill with 0 bits

◦ srl by i bits divides by 2i (unsigned only)

► Useful to mask bits in a word

◦ Select some bits, clear others to 0

and $t0, $t1, $t2

$t2

0000 0000 0000 0000 0000 1101 1100 0000

$t1

0000 0000 0000 0000 0011 1100 0000 0000

$t0

0000 0000 0000 0000 0000 1100 0000 0000

► Useful to include bits in a word

◦ Set some bits to 1, leave others unchanged

or $t0, $t1, $t2

$t2

0000 0000 0000 0000 0000 1101 1100 0000

$t1

0000 0000 0000 0000 0011 1100 0000 0000

$t0

0000 0000 0000 0000 0011 1101 1100 0000

► Useful to invert bits in a word

◦ Change 0 to 1, and 1 to 0

► MIPS has NOR 3-operand instruction

◦ a NOR b == NOT ( a OR b )

nor $t0, $t1, $zero

$t1 0000 0000 0000 0000 0011 1100 0000 0000 $t0

1111 1111 1111 1111 1100 0011 1111 1111 Register 0: always

read as zero

Instruction Example Meaning Comment and and $1,$2,$3 $1 = $2 & $3 3 reg. operands; Logical AND or or $1,$2,$3 $1 = $2 | $3 3 reg. operands; Logical OR xor xor $1,$2,$3 $1 = $2 ⊕ $3 3 reg. operands; Logical XOR nor nor $1,$2,$3 $1 = ~($2 | $3) 3 reg. operands; Logical NOR and immediate andi $1,$2,10 $1 = $2 & 10 Logical AND reg, constant or immediate ori $1,$2,10 $1 = $2 | 10 Logical OR reg, constant xor immediate xori $1, $2,10 $1 = $2 ⊕ 10 Logical XOR reg, constant shift left logical sll $1,$2,10 $1 = $2 << 10 Shift left by constant shift right logical srl $1,$2,10 $1 = $2 >> 10 Shift right by constant shift right arit. sra $1,$2,10 $1 = $2 >> 10 Shift right (sign extend) shift left logical sllv $1,$2,$3 $1 = $2 << $3 Shift left by variable shift right logical srlv $1,$2, $3 $1 = $2 >> $3 Shift right by variable shift right arit. srav $1,$2, $3 $1 = $2 >> $3 Shift right arith. by variable

► Example: lui R8, 255

31 26 25 21 20 16 15 0

001111

00000 01000 0000 0000 1111 1111 op rs rt immediate

► Transfers the immediate field into the register’s top 16 bits and fills the register’s lower 16 bits with zeros

R8[31:16] <-- 255 R8[15:0] <-- 0

31 16 15 0 R8

0000 0000 1111 1111

0000 0000 0000 0000

► We'd like to be able to load a 32 bit constant into a register

► Must use two instructions, new "load upper immediate" instruction

lui $t0, 1010101010101010

► Then must get the lower order bits right, i.e.,

ori $t0, $t0, 1111000011001010

filled with zero

31 16 15 0

1010 1010 1010 1010 0000 0000 0000 0000 1010 1010 1010 1010 0000 0000 0000 0000 ori

0000 0000 0000 0000 1111 0000 1100 1010 1010 1010 1010 1010 1111 0000 1100 1010

► Given an n-bit number

2x2xx = 1n - 1

0 0 1n -

+ 2n

-

2n

1 -

+  + 2x2x + ∎ Range: 0 to +2n – 1 ∎ Example

∎ 0000 0000 0000 0000 0000 0000 0000 = 0 + ... + 1×23 + 0×22 +1×21 +1×20

10112 = 0 + ... + 8 + 0 + 2 + 1 = 1110 ∎ Using 32 bits

∎ 0 to +4,294,967,295

► Complement and add 1

◦ Complement means 1 → 0, 0 → 1

xx +

= 1111...111 2 x1x

+ = - ∎ Example: negate +2

∎ +2 = 0000 0000 ... 00102 ∎ –2 = = 1111 1111 1111 1111 ... ... 110111102 2

+ 1

► Given an n-bit number

2x2xx = - 1n - 1

0 0 1n -

+ 2n

-

2n

1 -

+  + 2x2x + ∎ Range: –2n – 1 to +2n – 1 – 1 ∎ Example

∎ 1111 1111 1111 1111 1111 1111 1111 1100= – 0000 0000 0000 0000 0000 0000 0000 01002 2 = –410 ∎ Using 32 bits

∎ –2,147,483,648 to +2,147,483,647

► Bit 31 is sign bit

◦ 1 for negative numbers ◦ 0 for non-negative numbers

► –(–2n – 1) can’t be represented

► Non-negative numbers have the same unsigned and 2s-complement representation

► Some specific numbers

◦ 0: 0000 0000 ... 0000 ◦ –1: 1111 1111 ... 1111 ◦ Most-negative: 1000 0000 ... 0000 ◦ Most-positive: 0111 1111 ... 1111

Chapter 2 — Instructions: Language of the Computer — 41

► Representing a number using more bits

◦ Preserve the numeric value

► In MIPS instruction set

◦ addi: extend immediate value

◦ lb, lh: extend loaded byte/halfword

◦ beq, bne: extend the displacement

► Replicate the sign bit to the left

◦ c.f. unsigned values: extend with 0s

► Examples: 8-bit to 16-bit

◦ +2: 0000 0010 => 0000 0000 0000 0010

◦ –2: 1111 1110 => 1111 1111 1111 1110

Chapter 2 — Instructions: Language of the Computer — 42

Instruction Comment sw $3, 500($4) Store word sh $3, 502($2) Store half sb $2, 41($3) Store byte

lw $1, 30($2) Load word lh $1, 40($3) Load halfword lhu $1, 40($3) Load halfword unsigned lb $1, 40($3) Load byte lbu $1, 40($3) Load byte unsigned

lui $1, 40 Load Upper Immediate

(16 bits shifted left by 16)

∎ Example: sb $3, 12($1)

Immediate

Registers

000C

$1 $3 Data Bus

Memory . + . ALU .

Address Bus

. . .

∎ Example: lb $3, 12($1)

Immediate

Registers

000C

$1 $3 . +

. ALU .

Data Bus

Memory

Address Bus

. . .

Sign Extend

► Branch to a labeled instruction if a condition is true

◦ Otherwise, continue sequentially

► beq $rs, $rt, L1

◦ if (rs == rt) branch to instruction labeled L1;

► bne $rs, $rt, L1

◦ if (rs != rt) branch to instruction labeled L1;

► j L1

◦ unconditional jump to instruction labeled L1

Instruction Comment bne $t0, $t1, Label if (t0 != t1) goto Label beq $t0, $t1, Label if (t0 == t1) goto Label

► C code:

if (i==j) f = g+h; else f = g-h;

◦ f, g, ... in $s0, $s1, ...

► Compiled MIPS code:

bne $s3, $s4, Else add $s0, $s1, $s2 j Exit Else: sub $s0, $s1, $s2 Exit: ...

Assembler calculates addresses

► C code: while (save[i] == k) i += 1;

◦ i in $s3, k in $s5, address of save in $s6

► Compiled MIPS code:

Loop: sll $t1, $s3, 2 add $t1, $t1, $s6 lw $t0, 0($t1) bne $t0, $s5, Exit addi $s3, $s3, 1 j Loop Exit: ...

► Branch instructions end up the way we implement C-style loops

for ( j = 0; j < 10; j++) {

a = a + j; }

assume s0 == j; s1 == a; t0 == temp;

Instruction Comment addi $s0, $zero, 0 j = 0 + 0 addi $t0, $zero, 10 temp = 0 + 10 Loop: beq $s0, $t0, Exit if (j == temp) goto Exit add $s1, $s1, $s0 a = a + j addi $s0, $s0, 1 j = j + 1 j Loop goto Loop Exit: ... exit from loop and continue

► Set result to 1 if a condition is true

◦ Otherwise, set to 0

► slt $rd, $rs, $rt

◦ if ($rs < $rt) $rd = 1; else $rd = 0;

► slti $rt, $rs, constant

◦ if ($rs < constant) $rt = 1; else $rt = 0;

► Use in combination with beq, bne

slt $t0, $s1, $s2 # if ($s1 < $s2) bne $t0, $zero, L # branch to L

► Signed comparison: slt, slti

► Unsigned comparison: sltu, sltui

► Example

◦ $s0 = 1111 1111 1111 1111 1111 1111 1111 1111

◦ $s1 = 0000 0000 0000 0000 0000 0000 0000 0001

◦ slt $t0, $s0, $s1 # signed  –1 < +1 ⇒ $t0 = 1

◦ sltu $t0, $s0, $s1 # unsigned  +4,294,967,295 > +1 ⇒ $t0 = 0

R1= 0...00 0000 0000 0000 00012 = 110 R2= 0...00 0000 0000 0000 00102 = 210 R3= 1...11 1111 1111 1111 10002 = -810

► After executing these instructions: slt R4,R2,R1 ; if (R2 < R1) R4=1; else R4=0 slt R5,R3,R1 ; if (R3 < R1) R5=1; else R5=0 sltu R6,R2,R1 ; if (R2 < R1) R6=1; else R6=0 sltu R7,R3,R1 ; if (R3 < R1) R7=1; else R7=0

► What are values of registers R4 - R7? Why? R4 = ; R5 = ; R6 = ; R7 = ; 0 1 0 0

► Compare and Branch

◦ beq rs, rt, offset if R[rs] == R[rt] then PC-relative branch

◦ bne rs, rt, offset <>

► Compare to Zero and Branch

◦ blez rs, offset if R[rs] <= 0 then PC-relative branch

◦ bgtz rs, offset >

◦ bltz rs, offset <

◦ bgez rs, offset >=

► Remaining set of compare and branch take two instructions

Instruction Example Meaning set on less than slt $1,$2,$3 if ($2 < $3) $1=1; else $1=0 Compare less than; 2’s complement

set less than imm slti $1,$2,100 if ($2 < 100) $1=1; else $1=0 Compare < constant; 2’s complement

set less than uns. sltu $1,$2,$3 if ($2 < $3) $1=1; else $1=0 Compare less than; unsigned numbers

set l. t. imm. uns. sltiu $1,$2,100 if ($2 < 100) $1=1; else $1=0 Compare < constant; unsigned numbers

jump j 10000 go to (PC[31,28], 40000) Jump to target address

jump and link jal 10000 $31 = PC + 4; go to (PC[31,28], 40000) For procedure/subroutine call

jump register jr $31 go to $31 For switch, procedure/subroutine return

26 bits 6 bits

target address op J-type

► Constant data specified in an instruction

j 10000 ($PC <= $PC[31:28]:40000)

2 10000

► No subtract immediate instruction

◦ Just use a negative constant addi $s2, $s1, -1

► Design Principle 3: Make the common case fast

◦ Small constants are common

◦ Immediate operand avoids a load instruction

► Loop code from earlier example

◦ Assume Loop at location 80000

Loop: sll $t1, $s3, 2 80000 0 0 19 9 4 0

add $t1, $t1, $s6 80004 0 9 22 9 0 32

lw $t0, 0($t1) 80008 35 9 8 0

bne $t0, $s5, Exit 80012 5 8 21 2

addi $s3, $s3, 1 80016 8 19 19 1

j Loop 80020 2 20000

Exit: ... 80024

► If branch target is too far to encode with 16- bit offset, assembler rewrites the code

► Example

beq $s0,$s1, L1

↓ bne $s0,$s1, L2 j L1 L2: ...

6 6 bits

5 bits

5 bits

5 bits

5 bits bits R-type

op

rs

rt

rd

I-type op

rs rt immed Field Meaning op Basic operation of the instruction (opcode) rs First register source operand rt Second register source operand rd Register destination operand (gets result) shamt Shift amount funct Function field - selects the variant of the

operation in the op field (function code) immed Immediate value

shamt funct

6 bits 5 bits 5 bits 16 bits 6 bits 26 bits J-type

op target address

► Humans see instructions as words (assembly language), but the computer sees them as ones and zeros (machine language).

► An assembler translates from assembly language to machine language.

► For example, the MIPS instruction add $t0, $s1, $s2 is translated as follows:

Assembly Comment add op = 0, shamt = 0, funct = 32 $t0 rd = 8 $s1 rs = 17 $s2 rt = 18

000000

10001

10010

01000

00000 100000

op

rs

rt

rd

shamt

funct

► Addressing modes specify where the data used by an instruction is located.

► Often, the type of addressing mode depends on the type of operation being performed (e.g., branches all use PC relative)

► Five addressing modes are used in MIPS 3000:

◦ Register addressing

◦ Immediate addressing

◦ Base addressing

◦ PC-relative addressing

◦ Pseudodirect addressing

1. Register addressing (R-Type)

op rs rt rd ... funct

Register

Example:

add $s1, $s2, $s3 $s1 = $s2 + $s3

0 18 19 17 0 32

2. Immediate addressing (I-Type)

op rs rt immediate

Example:

addi $s1, $s2, 200 $s1 = $s2 + 200

8 18 17 200

3. Base addressing (I-Type)

op rs rt Address

Register + Byte Halfword Word Example:

lw $s1, 200($s2) $s1 = mem[200 + $s2]

35 18 17 200

Memory

4. PC-relative addressing (I-Type)

op rs rt Address

PC + Word Example:

beq $s1, $s2, 200 if ($s1 == $s2) PC = PC+4+200*4

4 18 17 200

Memory

5. Pseudodirect addressing (J-Type)

op Address

Memory

PC : Word Example:

j 4000 PC = (PC[31:28], 4000*4)

2 4000

Addressing Mode Summary

1. Immediate addressing

Immediate

2. Register addressing

oprsrt rd ... funct

Registers

Register

3. Base addressing | oprst Address

|

Memory

Register

Register

Byte Halfword

Word

4. PC-relative addressing

op rs rt Address

Memory

PC

Word

5. Pseudodirect addressing

ор

Address

Memory

-

PC

PC

Word

int A[100], B[100], C[100];

for (i=1; i < 99; i++) {

if (A[i] == B[i])

C[i] = A[i]; else C[i] = A[i-1]; } Assume base address of A is in $a0, B is in $a1, C is in $a2. Use $t0 for variable i.

Find bugs in this code and correct

addi$t0, $zero, 1 addi $t5, $zero, 99 loop:

lw $t1, 4($a0) lw $t2, 4($a1) beq $t1, $t2, equ lw $t1, 0($a0) equ: sw $t1, 0($a2)

addi $t0, $t0, 1 addi $a0, $a0, 4 addi $a1, $a1, 4 bne $t0, $t5, loop exit:

nop

► The MIPS assembler supports several pseudo-instructions:

◦ not directly supported in hardware

◦ implemented using one or more supported instructions

◦ simplify assembly language programming and translation

► For example, the pseudo-instruction

move $t0, $t1

is implemented as

add $t0, $zero, $t1

► The pseudo-instruction

blt $s0, $s1, Else

is implemented as

slt $at, $s0, $s1 bne $at, $zero, Else

► Register zero always has the value zero (even if you try to write it)

► Branch/jump and link put the return addr. PC+4 into the link register (R31)

► All instructions change all 32 bits of the destination register (including lui, lb, lh) and all read all 32 bits of sources (add, sub, and, or, ...)

► Immediate arithmetic and logical instructions are extended as follows:

◦ logical immediates ops are zero extended to 32 bits

◦ arithmetic immediates ops are sign extended to 32 bits

► The data loaded by the instructions lb and lh are extended as follows:

◦ lbu, lhu are zero extended

◦ lb, lh are sign extended

► Overflow can occur in these arithmetic and logical instructions:

 add, sub, addi

► it cannot occur in

 addu, subu, addiu, and, or, xor, nor, shifts, mult, multu, div, divu

► 32-bit fixed format inst (3 formats)

► 31 32-bit GPR (R0 contains zero) and 32 FP registers (and PC, HI, and LO)

► 3-address, reg-reg arithmetic instr.

► Single address mode for load/store: base+displacement

► Simple branch conditions

◦ compare one register against zero or two registers for =,≠